FUZZY LOGIC SYSTEM

Washing Machine Cycle Time Controller

Mamdani FIS  |  MATLAB Fuzzy Logic Toolbox  |  Introduction to Intelligent Systems

## 1. System Overview

| Field | Value |
| --- | --- |
| System Name | WashingMachineSystem |
| FIS Type | Mamdani (mamfis) |
| Course | Introduction to Intelligent Systems |
| Toolbox Required | MATLAB Fuzzy Logic Toolbox |
| Inputs (2) | Dirt Level (0–10), Load Size (0–10) |
| Output (1) | Wash Time (15–95 minutes) |
| MFs per Variable | 3 each (Low/Medium/High or Short/Medium/Long) |
| Rule Count | 9 rules (3 inputs × 3 inputs — full grid) |
| Output File | washing_machine.fis |

## 2. How Mamdani Fuzzy Inference Works

This system uses the classic Mamdani inference method, which processes inputs through four stages:

| # | Stage | What Happens |
| --- | --- | --- |
| 1 | Fuzzification | Each crisp input (e.g. DirtLevel = 6.5) is mapped to degrees of membership across all MFs using trapmf / trimf functions. |
| 2 | Rule Evaluation | All 9 rules are evaluated. The AND connector takes the minimum of the two input membership values to fire each rule. |
| 3 | Aggregation | The fired rule output MFs are truncated (clipped) at their firing strength and combined into a single output fuzzy set. |
| 4 | Defuzzification | The aggregate fuzzy set is converted to a crisp Wash Time in minutes using the centroid (centre-of-area) method. |

## 3. Inputs & Membership Functions

Input 1 — Dirt Level  [0, 10]

Represents how soiled the clothes are. Zero means clean; 10 means extremely dirty.

| MF Name | Shape | Parameters |
| --- | --- | --- |
| Low | trapmf | [0, 0, 2.5, 5] — flat top from 0 to 2.5, slopes off to 5 |
| Medium | trimf | [2.5, 5, 7.5] — peaks at 5, symmetric around centre |
| High | trapmf | [5, 7.5, 10, 10] — rises from 5, flat top from 7.5 to 10 |

Input 2 — Load Size  [0, 10]

Represents how full the drum is. Zero means empty; 10 means completely full.

| MF Name | Shape | Parameters |
| --- | --- | --- |
| Small | trapmf | [0, 0, 2.5, 5] — flat top at 0–2.5, slopes off to 5 |
| Medium | trimf | [2.5, 5, 7.5] — peaks at 5 |
| Large | trapmf | [5, 7.5, 10, 10] — rises from 5, flat top from 7.5 to 10 |

Output — Wash Time  [15, 95 minutes]

The crisp output represents the recommended wash cycle duration in minutes.

| MF Name | Shape | Parameters |
| --- | --- | --- |
| Short | trapmf | [15, 15, 35, 55] — flat from 15 min, slopes off to 55 min |
| Medium | trimf | [35, 55, 75] — peaks at 55 min |
| Long | trapmf | [60, 75, 95, 95] — rises from 60 min, flat at 75–95 min |

## 4. Rule Base (9 Rules)

All rules use AND (minimum) connectors with uniform weight 1. The full 3×3 grid is covered:

| # | IF Dirt Level | AND Load Size | THEN Wash Time | Weight |
| --- | --- | --- | --- | --- |
| 1 | Low | Small | Short | 1 |
| 2 | Low | Medium | Short | 1 |
| 3 | Low | Large | Medium | 1 |
| 4 | Medium | Small | Medium | 1 |
| 5 | Medium | Medium | Medium | 1 |
| 6 | Medium | Large | Long | 1 |
| 7 | High | Small | Medium | 1 |
| 8 | High | Medium | Long | 1 |
| 9 | High | Large | Long | 1 |

Design rationale:

- Low dirt always produces Short or Medium — load size is the differentiating factor.

- High dirt always produces Medium or Long — even a small load needs adequate washing.

- The centre cell (Medium + Medium → Medium) acts as the system's neutral baseline.

## 5. Code Section Walkthrough

Section 1 — FIS Construction

mamfis() creates a new Mamdani FIS object. Inputs and outputs are added with addInput() / addOutput(), specifying the universe of discourse. Membership functions are attached via addMF(), and rules are encoded as a numeric matrix passed to addRule(). The FIS is saved to disk with writeFIS().

fis = mamfis('Name', 'WashingMachineSystem');

fis = addInput(fis, [0 10], 'Name', 'DirtLevel');

fis = addMF(fis, 'DirtLevel', 'trapmf', [0 0 2.5 5], 'Name', 'Low');

Section 2 — Rule Matrix Format

Each row of ruleList encodes one rule as:  [Input1_MF  Input2_MF  Output_MF  Weight  Connection]

% Example: rule 6  →  Medium dirt AND Large load → Long wash

ruleList(6,:) = [2  3  3  1  1];

Connection 1 = AND (minimum operator). Weight 1 = fully weighted rule.

Section 3 — Visualisation

Three types of plots are produced:

| Figure | Purpose |
| --- | --- |
| Figure 1 (3 subplots) | plotmf() — shows all MFs for Dirt Level, Load Size, and Wash Time in a single window |
| Figure 2 (Surface) | gensurf() — 3-D control surface showing Wash Time over the full Dirt × Load input space |
| Rule Viewer | ruleview() — interactive GUI for manually testing input combinations and inspecting rule firing |

Section 4 — Test Cases

evalfis() evaluates crisp inputs against the trained FIS. The 6 test cases cover corners, centre, and cross-diagonal scenarios:

| Case | Dirt | Load | Expected Range | Scenario |
| --- | --- | --- | --- | --- |
| 1 | 2 | 2 | Short (~30–40 min) | Clean clothes, small load |
| 2 | 5 | 5 | Medium (~55 min) | Average everything |
| 3 | 8 | 8 | Long (~75–85 min) | Dirty, full load — worst case |
| 4 | 3 | 7 | Medium–Long | Clean but big load |
| 5 | 7 | 3 | Medium | Dirty but small load |
| 6 | 5 | 9 | Long | Average dirt, oversized load |

## 6. Issues & Recommendations

| # | Issue | Detail | Recommendation |
| --- | --- | --- | --- |
| 1 | Output MF gap (Short vs Medium) | Short MF ends at 55; Medium trimf peaks at 55 and starts at 35. Long starts at 60. There is a gap between 55 and 60 with partial coverage only. | Extend Short trapmf to [15, 15, 35, 60] or adjust Medium to cover 55–60 fully. |
| 2 | ruleview() may block script | The interactive rule viewer GUI can stall script execution in some MATLAB environments or when running non-interactively. | Move ruleview() to the very end, or wrap in try/catch / conditional on isdeployed(). |
| 3 | No aggregate metrics printed | Individual test results are printed but no summary (min, max, mean wash time across cases) is reported. | Add: fprintf('Mean: %.1f Min: %.1f Max: %.1f\n', mean(results), min(results), max(results)); |
| 4 | MF parameter symmetry inconsistency | DirtLevel and LoadSize MFs use identical parameters, but the output (WashTime) Short MF [15,15,35,55] is wider than Long [60,75,95,95], creating asymmetric coverage. | Review whether Short should be [15,15,30,50] to match the symmetric structure of Long. |
| 5 | No input validation | evalfis() will silently extrapolate if test inputs fall outside [0,10]. No guard exists. | Add: assert(all(testCases(:) >= 0 & testCases(:) <= 10), 'Input out of range'); |

## 7. Output Files & MATLAB Requirements

| Output / Artifact | Description |
| --- | --- |
| washing_machine.fis | Exported FIS — reload with: fis = readfis('washing_machine') |
| Figure 1 (MF subplots) | 3-panel membership function display for both inputs and the output |
| Figure 2 (Surface) | 3-D gensurf() control surface — Wash Time across full input space |
| Console table | Formatted fprintf output of 6 test case results |
| Rule Viewer GUI | Interactive ruleview() window for manual tuning and inspection |

Required MATLAB Toolbox:

- Fuzzy Logic Toolbox — mamfis, addInput, addOutput, addMF, addRule, evalfis, writeFIS, plotmf, gensurf, ruleview

## 8. Possible Extensions

- Add WaterTemperature (0–10) for a 3-input, 27-rule system.Third input:

- Collect real sensor data and train an ANFIS on top of this FIS for adaptive learning.ANFIS upgrade:

- Add a second output for Spin Speed (RPM) using a separate FIS or MISO extension.Additional output:

- Experiment with 'bisector' or 'mom' defuzzification methods and compare Wash Time outputs.Defuzzification:

- Adjust MF widths using trial test cases; plot error sensitivity to MF boundary shifts.Parameter tuning:

Technical documentation — Fuzzy Logic Washing Machine Controller